SANDBOX · PROFILE STUDIO — TUTORIAL 2

Using a CSV

Define an arbitrary, curved profile from a width/height trace instead of the rectangular Space opening. A CSV can describe a whole etched profile, not just a simple opening.

| The CSV format Two columns, one row per point: • width — the full opening width (CD) at that height. A size, always ≥ 0. • height — the vertical position in nm. May be negative (it's a position). • The profile is drawn symmetric about the centerline (±width/2). width; height. 0; 0. 22; 6. 36; 16. 48; 40. 56; 80. 62; 130. 64; 180. 62; 220. 58; 248. 60; 262. example_profile.csv — a realistic etched trench |  | The trace loaded as the profile |
| --- | --- | --- |

▍ Steps

1.  Prepare a CSV with columns width,height (see example). Save it anywhere.

2.  In the OPENING FROM CSV section, click “Load CSV…” and choose your file.

3.  Placement. By default the trace is top-aligned (its top sits at the stack top, cutting downward). To pin it elsewhere, set “CSV height=0 at (nm)” — the row where height = 0 lands that many nm from the stack bottom; negative heights sit below that line.

4.  While a CSV is active, Space and Opening depth are ignored (they gray out); Top vacuum still applies.

5.  If the curved walls look jagged, raise Smoothing (Off / 2× / 4× / 8×). The preview updates live — click Save .bmp… to export.

Tips • If the material stack is empty, a silicon base is added automatically so the profile is visible. • Negative width is rejected; ragged rows or blanks are rejected with a clear message. • Points don't need to be pre-sorted — they're ordered by height internally.