SANDBOX · PROFILE STUDIO — TUTORIAL 3

The process stack

After the material stack and opening are built, the process stack transforms the profile step by step — the same idea as real fab steps (deposit, fill, etch, polish).

▍ How it works

Steps run in order, top to bottom (①, then ②, ③ …). Each step acts on the result of the one above it. A Repeat block runs its steps ×N — handy for multilayers / superlattices.

▍ The step types

•  Deposit · conformal — coats every surface (walls, bottom, top) with a uniform layer that follows the profile. Use it for liners and spacers.

•  Deposit · planar — adds a flat layer on top only (no sidewall coverage).

•  Fill — gap-fills the opening flush with the surface. The number is overfill (extra nm of blanket above the surface; 0 = flush).

•  Etch · isotropic — removes material equally in all directions; rounds corners and undercuts.

•  Etch · anisotropic — removes material vertically. Anisotropy 1 = fully vertical, 0 = isotropic, in between = a rounded undercut. Pick a material to etch only that one, or “(any)”.

•  Planarize — cuts everything flat at a height, like chemical-mechanical polish (CMP).

▍ Worked example — liner + fill + polish

| ① bare trench (stack + opening) |  | ② ＋ Deposit·conformal (nitride liner) |  | ③ ＋ Fill (tungsten) ＋ Planarize |
| --- | --- | --- | --- | --- |

Tips • Deposit / Fill can use ANY material in the palette — you don’t need to add liner/fill materials to the stack first. Etch lists the materials present (plus “(any)”). • Etch with a material selected removes only that material; “(any)” removes whatever it reaches. • Build a superlattice with a Repeat block: e.g. deposit A, deposit B, repeated ×10.